RUNOFF (.rff) and RECHARGE (.rch) file creation  (March 22, 2003 nk)

These files are created to allow WATFLOOD to be linked to other software or models.

To create these files:

- Set flag #9 (col. 59) in the event file = ‘y’

- Create a runof and grndw subdirectories in the working directory e.g. spl\gr10k\runof and spl\gr10k\grndw

- Provide names for files no. 16 & 17  in the event file as shown below.

Note: The reason the files are not sent to the simout directory is that they are input files for other models and are part of the information flow of the modeling.

The .rff file is a file of hourly grids of the sum of surface runoff and interflow. It is the direct runoff resulting from rainfall or snow melt. It is file number 16 in the event file. It is formatted to be read by WATROUTE. The units are m3/s.

The .rch is an output file formatted for MODFLOW. Care must be taken that the grids will coincide. Currently the output is the 24 hour recharge. When SPL is run in this mode, the water is added to the lower zone storage as usual. If the user would like to run SPL with the lower zone outflow (leakage) turned off, simply set the LZF = -ve in the parameter file.

Example of an EVENT file to create runoff and recharge files with the relevant entries bolded:

date:                     93 01 01 00   ynynnnnnynn

month:                        1   04**

event\930201.evt

event\930301.evt

event\930401.evt

rain conv. factor @ scale  1.00  .00

ini moist each subbasin   -1.00-1.00-1.00-1.00-1.00

no hours of rain data       744

no hours of flow data       744

basin file name           BASIN\gr10k.SHD

parameter file name       BASIN\gr10k.PAR

rain gauge locn file name BASIN\gr10k.RAG

stream gage locn file nam BASIN\gr10k.STR

rain gauge data file  name RAING\930101.RAG

streamflow data file      STRFW\930101.STR

reservoir release file    RESRL\930101.REL

snow data file            SNOW1\930101.SNW

radar met file            RADUC\930101.RAD

simple input met file     RADCL\930101.MET

AES hourly radar rainfall RADAR\930101.scn

clutter file              RADAR\930101.clt

snow cover depl. curve    basin\gr10k.sdc

point temps               tempr\930101.tag

gridded temps             tempr\930101.tem

Gridded runoff            runof\930201.rff

Gridded recharge          grndw\930201.rch

dsn file                  snowg\930101.dsn

** Up to 100 events can be chained by entering the # of events to be chained in col. 32-36
The event names for the series of events to be modelled are then listed as shown.

There are 12 flags:

|  | Flag: | Result if ‘y’ |
| --- | --- | --- |
| 1 | Snwflg | snowmelt routines will be used |
| 2 | Sedflg | sediment production and routing routines will be used |
| 3 | Vapflg | Evaporation turned on (need temperature files) |
| 4 | Smrflg | Precip. data will be smeared - e.g., precip. entered once every 24 hours will be ‘smeared’ over the whole day instead of taken as an hourly amount |
| 5 | Resinflg | reservoir inflow data required and computed reservoir inflows will be compared |
| 6 | Tbcflg | a resume.txt file containing all state variable values at program termination will be written |
| 7 | Resumflg | the resume.txt file will be used to initialize state variables - allows the program to resume a time series as if it was executed as a continous run |
| 8 | Contflg | continue the statistics from previous run via resume.txt file |
| 9 | Routeflg | write the \spl\bsnm\runoff\yymmdd.txt for watroute.exe and \spl\bsnm\grndw\yymmdd.txt |
| 10 | crseflg | read snow course data to replace resume file data |
| 11 | Ensimflg | Create a \spl\simout\watflood.wfo file for ENSIM |
| 12 | Wetflg | Use coupled wetland-channel routing |