Section 6.2

Reservoir Release File   1/29/02

- The XXX.REL file has the reservoir locations and releases.

3   90    1

4843  554   0.00000   0.00000   0.00000   0.00000   0.00000  Belwood

4836  523   0.00000   0.00000   0.00000   0.00000   0.00000  Conestogo

4827  559   0.00000   0.00000   0.00000   0.00000   0.00000  Guelph

4.900     9.940     4.400

4.900     9.940     4.400

4.900     9.940     4.400

.

.

.

46.300    94.700    60.000

46.300    94.700    60.000

46.300    94.700    60.000

46.300    94.700    60.000

The first number is NR, the number of reservoirs and lakes; the second is NHR, the number of hours of release information; and the third number is the time step, which has to be the same as the time step for the streamflow information. (This is automatically taken care of when a new event is initiated).

The next NR lines give the locations, operating rule and name for each reservoir. The location is in UTM coordinates. The operating rule has to be programmed for each individual reservoir but five parameters are reserved for this purpose. In the current version of SPL9, the releases are externally determined and entered by way of the WATFLOOD menu. The remaining values are the releases in cms. Values are not required for each time step. If there is a negative value, the last positive value is carried forward by the program.

IRES JRES        B1        B2        B3        B4         B5    comment

4843  554         0         0         0         0         0     SHAND

4836  523         0         0         0         0         0     CONESTOGA

Format:   2I5, 5g10.3

IRES and JRES are the coordinates of the reservoir outlet structures. B1 to B5 are parameters available for a reservoir or lake routing relationship if the releases are not read in.

Lake Routing

A lake can be modelled using a two step procedure. First mark each grid that is part of a lake with a reach number in the map file. Number the lakes from 1 to the number of lakes. If a lake covers all or part of multiple adjoining grids, mark each grid touched by that lake with the same reach number. Reservoirs with controlled outlets should also be marked as lakes and should be placed ahead of the naturally controlled outlets. See Section Error: Reference source not found for an example of setting up the reach numbers in the bsnm.map file. Once the lakes have been located, the outlets should be located in the outlet grid and entered into the yyddmm.rel file as shown in Sect. Error: Reference source not found.

Water is routed through the lakes using a user-specified function. Either a power function

Outflow= b1 Storageb2

or a polynomial like

Outflow=b1∗storage+b2∗storage2+b3∗storage3+b4∗storage4+b5∗storage5

- must be used. If  b3, b4 and b5 =0.0, a power function with coefficients b1 and b2 is assumed. If b3 or b4 or b5  0.0, a polynomial is assumed. For the latter, b3 must have a value although b4 and/or b5 can be 0.0. However, it is very important that the coefficient of the highest order term is +ve.

If a power function provides the best fit, only the first two parameters are used (B1 and B2). If a polynomial is used, it must be a 3, 4 or 5 parameter polynomial. It is important that the polynomial be monotonically increasing and that it does not dip down after the last point. For this reason, the coefficient for the highest order term must be positive. A 3rd, 4th or 5th order function can be tried and the best one meeting these requirements can be chosen. Sometimes extra points added to the data set can be used to force the function to behave. The function MUST be plotted to ensure is monotonically increasing and passes through the origin (0,0) of the graph.

An example input file is shown below.

Below is an example for Tabacco Creek for a watershed with any farm ponds. An Excel spreadsheet is used to fit polynomials or power functions to each of the storage-discharge curves.

Figure 6.1 Example of a storage-discharge curve (Subbasin 55 below)

(Please note that the order of the terms is reversed below)

Example yyyymmdd.rel file:

22  744    1    y

5462  545 6.051E-05 1.265E-09-4.103E-13 1.397E-17 0.000E+00     Subbasin 56

5462  548 5.947E-06-9.931E-11 1.079E-15-1.223E-20 7.798E-26     Subbasin 55

5462  549 4.062E-10 4.800E-10-1.711E-14 1.732E-19 0.000E+00     Subbasin 53

5463  545 1.502E-04-2.096E-08-1.258E-12 5.765E-17 5.464E-20     Subbasin 50

.

.

5469  545 3.720E-04-1.505E-07 2.722E-11 0.000E+00 0.000E+00     Subbasin 13

5471  542 2.290E-02 2.212E-01 0.000E+00 0.000E+00 0.000E+00     Subbasin 5

5471  544 3.334E-08 1.537E+00 0.000E+00 0.000E+00 0.000E+00     Subbasin 6

5471  545 1.131E-04-1.711E-08 1.515E-12-8.503E-17 2.418E-21     Subbasin 4

- For controlled reservoirs, the releases must be entered in the resrl\yymmdd.rel file. The controlled reservoirs are indicated by b1 and b2 = 0.0 in the header of the yymmdd.rel file.

For geographical coordinates, the lake or reservoir locaton is given in minutes as for the Ticino River in Italy:

1   24    1

2744  516 0.100E-09  1.75     0.000E+00 0.000E+00 0.000E+00      Maggioro

fln = Section 6.2.doc