Section 3.3.11

Routing Reach Number (IREACH)     1/29/02

In some situations, the user may wish to route flows outside the SPL program, for instance where back water or tidal effects have to be taken into account. For this purpose, a reach number can be inserted for those elements where channel inflows are desired as output in a separated file. The output will be in DWOPER format 8F10.3 in the \spl\simout\rte.lst file. A block of zeros is required where there is no external routing.

This block is also used to group elements into one or a number of reservoirs or lakes to allow an accounting of reservoir storage and/or reservoir level reporting. (See section on Lake routing).

Reach Number (IREACH)

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 1 0 0 0

0 0 0 0 0 1 0 0 0

0 0 2 0 0 0 0 0 0

0 0 0 0 0 3 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 0

- Format:   999I2

In this example there are three reservoirs. The numbers 1 to 3 correspond to the reservoir locations in the resrl\yymmdd.rel file. In this example, the Belwood reservoir (No. 1) is located in two grids (not in reality) and the outlet id in the bottom grid.

fn = ROUTING_REACH_NUMBER.doc